Anti-Skid Spray

The purpose of this procedure is to understand how to use the anti-skid spray by Gorilla Truck Bed Coating. This is available at your local Walmart or through Amazon. Before starting the application process the WH Manager will verify and communicate with the Fleet Manager to ensure the truck is not slated to be sold.

Steps for Preparation

- Start by making sure that your workspace is well-ventilated. Use appropriate protective gear, including gloves, glasses, and a mask.

- Wipe off any dust.

- Use a degreaser or warm soapy water to wash the area where you want to apply the Gorilla Truck Bed Coating aerosol spray if the area is overly dirty. Allow it to dry.

- Frame the application area with tape. Cover any surface around the area with paper or plastic sheeting.

Steps for Applying

- Shake the can to make sure the paint is ready to be applied.

- Before applying to chosen surface spray a bit on cardboard to test the texture

- Hold the can upright about 12” away from the surface.

  - For a coarser texture, hold the can slightly closer to the surface.

  - For a finer texture, hold the can slightly farther away.

- Use a sweeping motion to give the area a light first coat.

- Wait 10-15 minutes.

- Repeat steps 2-3 to apply a second coat.

- Allow the surface to cure for 3-4 hours before use

Annual Inspection

Trucks will be inspected annually by Warehouse Managers to check durability and where the process will be reapplied following the above steps.